# TRRP Supplemental Metadata for Trinity River Flow Data

# Title

Trinity River Flow Data, WY2015 edition, version 2.

# FGDC-compliant Metadata files

TrinityRiverFlowDataWY2015.xml

ReservoirOperations\DamOps.shp.xml

ReservoirOperations\HAR_PoR_thru_WY2015.xls.xml

RestorationFlowReleaseSchedule\RestorationFlowReleaseSchedule.shp.xml

RestorationFlowReleaseSchedule\RestorationFlowSchedules.csv.xml

Streamgages\Trinity_Watershed_Gages.shp.xml

Streamgages\TRStreamgages_WY2015.xml

# Recommended Citation

Trinity River Restoration Program (TRRP). 2016. Trinity River flow data, WY2015 edition, version 2. TRRP Data Package.  Trinity River Restoration Program, Weaverville, CA. Online at http://odp.trrp.net.

# Introduction

This Data Package provides an organized set of time-series reference data relevant to flows in the Trinity River of northern California with geospatial data representing sample locations.  The data are organized into 3 subsets:

- Streamgages.  An organized compendium of streamgage data throughout the Trinity River watershed (plus the Klamath River above and below the confluence with the Trinity).  Data include full historic records, instantaneous stage and flow records where available or where have been specifically obtained by TRRP, and streamgage sites that no longer operate.  Streamgage data in this data package are primarily from the USGS and are obtainable online from their National Water Information System (NWIS) or their Instantaneous Data Archive (IDA). Data also include some tributary data collected by agencies or contractors that are not currently available from USGS.

- Reservoir Operations.  A single table with reservoir data on storage and release beginning November 1962 (the Trinity River Division dams began retaining water in late January, 1961).

Figure 1: Streamgaging sample design for flows on the Trinity River from Lewiston Dam to the North Fork Trinity River.

Restoration Flow Release Schedule. A single table with the scheduled daily release from Lewiston Dam recommended for restoration purposes in the Trinity River.  Prior to WY2007, restoration flows were typically subject to limitations and should not be characterized simply as ‘TMC recommended restoration flow schedules’.

  - WY2001 – WY2004 scheduling was limited by court order

  - WY2005 – WY2006 maximum flow rate for scheduling was limited by infrastructure.

More details on restoration flow releases are available in annual reporting of flow releases (e.g. TRRP 2014).

Water years are defined as October 1st through September 30th.  Version/Edition names will use the water year for which data was recently added.

This 3-part ‘flow’ data package supersedes prior ‘streamgage’ data packages, which only contained streamgage data. The "WY2014b" edition removes a glitch in the Reservoir Operations subset, Trinity Reservoir Calculated Inflows (TRN.QI) for part of WY2014.

# Methods

## Sampling Design

Streamgages. In general, the U.S. Geological Survey should be consulted on sampling design for streamgages in the Trinity River watershed.

For the 64km of the Trinity River directly below Lewiston Dam (Lewiston Dam to the North Fork Trinity River), TRRP has worked with the USGS to design sampling such that contributions of major tributaries can be approximated individually, or in small sets.  Figure 1 provides guidance.

Reservoir Operations. Both reservoirs are represented in the data requested by TRRP.  The data parameters requested are those most relevant to the inflow and river release.

Restoration Flow Release Schedule.  A sampling design is not applicable.

## Collection Protocol

Streamgages. In general, the U.S. Geological Survey should be consulted on data collection protocols for streamgages in the Trinity River watershed.  For the streamgage data not available through the USGS, see GMA (2007).

GIS point locations were generated from coordinates provided by the U.S. Geological Survey.  In a few cases, these point locations have been refined to more accurately place the gage location; these are flagged in the attribute table (POS_ADJ = 1).

Reservoir Operations. The U.S. Bureau of Reclamation, Central Valley Operations, should be consulted on data collection protocols.

GIS point locations were placed in a central location over the dams as viewed in aerial photography.

Restoration Flow Release Schedule. A data collection protocol is not applicable.

GIS point location was placed over the pool at the base of the Lewiston Dam spillway.

## Quality Assurance

Streamgages.  Data are not obtained from USGS until listed as ‘approved’.  Data are provided as received from the USGS. For data not available from the USGS, see GMA (2007).

Reservoir Operations.  Data are obtained from the U.S. Bureau of Reclamation, Central Valley Operations after the end of the water year, no earlier than the following February, to allow CVO time for Quality Control. Data are then copied/pasted into the table for the full period of record with verification that the column order matches.  Data from the California Data Exchange Center (CDEC) are not used, as they are stored prior to quality-control and may not reflect changes after quality control.

Restoration Flow Release Schedule.  Data are obtained through the coordinator of the Flow Workgroup, and copied/pasted into the table for the full period of record.

## Quality Control

Quality control is primarily performed by the data source office and staff.  Additionally, reservoir and flow scheduling data are plotted within Microsoft Excel to verify logical values.

## Analysis / Data Processing

Not applicable to this data package.

# Results/Discussion

## Assumptions, Caveats, Use Considerations

- Streamgages.

  - Note that the USGS rates streamgage data quality.  For modern streamgaging, accuracy of +/- 5 to 10 percent is common.

  - Older data and non-USGS data may be subject to greater accuracy challenges.

- Reservoir Operations.

  - Data are subject to change as Quality Control issues are identified.

- Restoration Flow Release Schedule.

  - Schedules are daily values.  These are used by Central Valley Operations to schedule changes to dam releases, which may include several changes per day to accommodate ‘ramping rates’ in the Trinity River Final Environmental Impact Statement, Appendix C (USFWS et al. 2000), or other operational considerations.

  - Recommended schedules are subject to approval by the U.S. Bureau of Reclamation and U.S. Fish and Wildlife Service as CVPIA co-lead agencies for the U.S. Department of Interior, which has the final authority over dam releases.

  - Prior to WY2007, restoration flows were typically subject to limitations and should not be characterized simply as ‘TMC recommended restoration flow schedules’.

    - WY2001 – WY2004 scheduling was limited by court order

    - WY2005 – WY2006 maximum flow rate for scheduling was limited by infrastructure.

## Data Organization

General. The parent folder of this data package contains a few files that apply to all components:

- A master metadata XML file that applies to the whole data package.

- This supplemental metadata document (in DOC and PDF formats).

- An ESRI ArcGIS project file for viewing data locations in a mapping environment.

The parent folder also contains 3 subfolders, one for each of the data subsets.

Streamgages.

Geospatial locations of streamgages are given in Trinity_Watershed_Gages.shp and related files.  Attribute definitions:

| Attribute | Units | Definition | Notes |
| --- | --- | --- | --- |
| USGS_CODE |  | US Geological Survey 8-digit identifier for gage |  |
| USGS_NAME |  | US Geological Survey abbreviated name as used in NWIS |  |
| ODP |  | Hyperlink to data for gage in the TRRP Online Data Portal |  |
| WYFirst |  | First water year with data |  |
| WYLAST |  | Last water year with data | Zero for currently operating gages |
| YEARSCONTI |  | Years of continuous measurement | May include gaps and notes on continuity |
| ACTIVE |  | Identifies currently operating gages | Yes/No |
| NWIS |  | Hyperlink to overview page for the gage in NWIS |  |
| NWIS_GRAPH |  | Hyperlink to the data graph page in USGS NWIS |  |
| IDA |  | Hyperlink to the gage in the USGS Instantaneous Data Archive |  |
| USGS_X |  | Longitude as provided by USGS NWIS |  |
| USGS_Y |  | Latitude as provided by USGS NWIS |  |
| USGS_Z |  | Elevation as provided by USGS NWIS |  |
| USGS_HORIZ |  | Horizontal datum as stated by USGS NWIS |  |
| USGS_VERT_ |  | Vertical datum as stated by USGS NWIS |  |
| POS_ADJ |  | Flag to identify points where the placement has been adjusted by TRRP to better match the gage position | Typically done with a combination of field experience and high resolution aerial photography |
| NWIS_Peaks |  | Hyperlink to peak flow data in USGS NWIS |  |

Streamgage time-series data are organized into subfolders first by status of the gage, then by USGS gage number. Folder names also include indicators of the types of data contained, and the abbreviated USGS name for the gage.  Data type indicators are: (q) daily flow, (i) instantaneous flow, (p) annual peak flow, (s) stage, and (t) temperature (when collected by the USGS along with gage data; most temperature monitoring on the Trinity River operates separately).

- Gages-Active

  - 11523000_qipst_KLAMATH_R_A_ORLEANS

  - 11523200_qips_TRINITY_R_AB_COFFEE_C_NR_TRINITY_CENTER_CA

  - …

- Gages-Historic

  - 11523100_--p-_DAN_RICE_C_NR_CALLAHAN_CA

  - 11523700_q-p-_COFFEE_C_NR_TRINITY_CENTER_CA

  - …

Within these subfolders, data consist of numerous files in several formats.  Filenames include the following conventions.

- dvq (or just q) – mean daily discharge,

- uvq (or i) – instantaneous discharge, typically at 15 minute intervals

- uvqs – instantaneous discharge and stage (USGS now provides these together in downloads of data from recent years)

- uvqst – as with uvqs above, but with additional data on water temperature.

- peak (or just p) – annual peak discharge

- uvs (or just s) – instantaneous stage value

- WY – water year, running October 1 through September 30.  The year number matches the calendar year that occupies the majority of the water year, thus WY2008 refers to 10/1/2007 – 9/30/2008.

File types and formats include

- USGS Data

  - .rdb and .txt - Most data acquired from USGS are in their rdb format (files ending either .rdb or .txt).  Many data users will be familiar with this text based format from the NWIS websites.  Some metadata is included at the top of the file.  The format uses a UNIX/Linux carriage return that typically does not display well in MS Windows Notepad, but does display OK with MS Windows Wordpad.  There are several freely-downloadable text file viewers that will display these files correctly (e.g. Notepad++).  Columns are tab delimited, and can be imported into spreadsheets and databases with minimal effort.

  - Naming convention within the data package attempts to specify gage number, data content, and timespan.

  - Note that time values jump between standard time and daylight savings.

  - .xls or .csv – Some special data requests to the USGS have been supplied with .xls or .csv files.  In these cases, the filename is prefixed with “USGS_” and the remainder of the filename follows the convention specified above.

- Non-USGS Data

  - Several datasets acquired from non-USGS gage operations were imported into IIMS (now the Online Data Portal; ODP), but now lack original data files within the TRRP Weaverville office fileserver.   For each of these, four files are contained within the data package:

  - 4IIMSin2007_[].xls – Excel file quality controlled by Andreas Krause in January, 2007, for importation into the IIMS database.

  - 4IIMSin2007_[].csv – CSV file quality controlled by Andreas Krause in January, 2007, for importation into the IIMS database.

  - ODPin2012_[].csv – CSV file downloaded from ODP by Eric Peterson in March, 2012.

  - []_README.txt – brief description of gage operation

Any datasets derived from the baseline datasets are prefixed with the word DERIVED.  Their filename or notes within the file will indicate their purpose.

An R Script is now included within the data folder for the Lewiston gage.  This script can be used with the R statistical package (http://cran.r-project.org/) to generate graphs of flow over time at the Lewiston gage.  This graph is commonly known as the ‘pulse of the river’ graph, as it is displayed for public at the TRRP office in Weaverville.

Reservoir Operations.

Geospatial locations are given in DamOps.shp and related files.  Attribute definitions:

| Attribute | Units | Definition | Notes |
| --- | --- | --- | --- |
| Name |  | name of the reservoir dam |  |
| WSE | feet above sea level | link to reservoir water surface elevation time series data in the TRRP Online Data Portal |  |
| Storage | acre feet (af) | link to calculated reservoir storage volume time series data in the TRRP Online Data Portal |  |
| Inflow | cubic feet per second (cfs) | link to reservoir inflow time series data in the TRRP Online Data Portal |  |
| TotRelease | cubic feet per second | link to total reservoir dam release time series data in the TRRP Online Data Portal. | Trinity Reservoir only |
| Precip | inches | link to precipitation as measured at the dam time series data in the TRRP Online Data Portal. | Trinity Reservoir only |
| Evap | cfs | : link to calculated evaporation from the reservoir time series data in the TRRP Online Data Portal. | Trinity Reservoir only |
| ReleaseRiv | cfs | link to total in-river release time series data in the TRRP Online Data Portal. | Lewiston Reservoir only |
| DiversCarr | cfs | link to diversion flow to the Carr Power Plant (Whiskeytown Reservoir) and thus the Central Valley time series data in the TRRP Online Data Portal. | Lewiston Reservoir only. |

Tabular time-series data is provided in HAR_PoR_thru_WY2014.xls. Field definitions:

| Field | Units | Definition | Notes |
| --- | --- | --- | --- |
| Date | Day | Date in native Excel format | Displayed as month/day/year |
| LEW.HL | Feet above sea level | Reservoir elevation |  |
| LEW.LS | af | Reservoir storage |  |
| LEW.QI | cfs | Computed inflow |  |
| LEW.QR | cfs | Release to river |  |
| JCR.QG | cfs | Generation release | quantifies water diverted to the Central Valley |
| TRN.QI | cfs | Computed inflow |  |
| TRN.HL | Feet above sea level | Reservoir elevation |  |
| TRN.LS | af | Reservoir storage |  |
| TRN.QT | cfs | Total release | to Lewiston Reservoir |
| TRN.PP | Inches | Precipitation | as measured at dam |
| TRN.ES | cfs | Calculated evaporation |  |

FGDC compliant metadata are included for both geospatial and time-series data files; those metadata reference this supplemental document for most information pertinent to data use.

Restoration Flow Release Schedule.  .

Geospatial locations are given in RestoratationFlowReleaseSchedule.shp and related files.  Attribute definitions:

| Attribute | Units | Definition | Notes |
| --- | --- | --- | --- |
| Name |  | name of the data |  |
| RestFowSc | cfs | link to restoration flow release schedule time series data in the TRRP Online Data Portal |  |

Tabular time-series data is provided in RestorationFlowSchedules.csv (comma separated values). Field definitions:

| Field | Units | Definition | Notes |
| --- | --- | --- | --- |
| Date | Day | Date | Displayed as month/day/year |
| CFS | cfs | Recommended daily flow release |  |

FGDC compliant metadata are included for both geospatial and time-series data files; those metadata reference this supplemental document for most information pertinent to data use.

# Acknowledgements

The U.S. Bureau of Reclamation, Central Valley Operations, provides the reservoir operations data.  The U.S. Geological Survey provides most of the streamgage data; in most cases these are conveniently accessible through their online databases.

# References

Graham Matthews and Associates (GMA). 2007. Trinity River comprehensive sediment transport data compilation report. Report to the Trinity River Restoration Program by Graham Matthews and Associates, Weaverville, CA.

Trinity River Restoration Program (TRRP). 2014. Restoration Flow Releases on the Trinity River – Water Year 2013. Technical Report #TR-TRRP-2014-1. Trinity River Restoration Program, Weaverville, CA.

U.S. Department of Interior (USDI). 2000. Record of decision, Trinity River mainstem fishery restoration final environmental impact statement/environmental impact report. Decision by the U.S. Department of Interior, December 2000.

U.S. Fish and Wildlife Service, U.S. Bureau of Reclamation, Hoopa Valley Tribe, and Trinity County. 2000. Trinity River Mainstem Fishery Restoration Environmental Impact Statement / Environmental Impact Report. Public Draft and Final in electronic format.

Version 2 corrects a few days of values for reservoir release (QR) from Lewiston Dam around the peak of the spring restoration release.

Date approximated by comparing the ratio of flows between the Lewiston gage and the Coffee Creek gage.  Some unusual ratios are visible as early as November, 1960, but only for a few days duration.  Although water was being retained, the dams were not considered fully operational for a couple more years.