- Sediment mobility

- Means Objective: Maintain sediment mobility at thresholds that aide physical and biological processes.

- Fundamental Objective: Promote channel complexity, intergravel flow, and river meandering.

- Target: Maintain critical Shields stress for the median grain size (τc50*) between 0.025 and 0.085.

- Localities: Sediment monitoring transects at Lewiston, Limekiln Gulch, and Douglas City.

- Frequency and Timing: During water years designated as dry and wetter with ROD releases >4,000 cfs.

- Methodology: With shear stress partitioned for the granular bed, extrapolate physically sampled mass transport rates of bed load size fractions, Di, ≥0.5 mm to a dimensionless reference value of 0.002 for each Di. Compute fractional Shields stresses, τci*, for Di and plot against Di/D50, where D50 is sampled on the bed surface near the respective sediment monitoring stations prior to the spring high flow release. Use the hiding function of Parker et al. (1982; τci*=τc50* DiD50b) to estimate τc50* with power functions of τci* on the ordinate versus Di/D50 on the abscissa.

- Fine sediment storage

- Means Objective: Promote coarse sediment mobility, riparian vegetation recruitment, lamprey populations, and groundwater storage.

- Fundamental Objective: Maintain fine sediment storage at levels that promote healthy river functioning.

- Targets: (1) Maintain the exponent (b) in the hiding function of Parker et al. (1982;

- τci*=τc50* DiD50b)τc50*) at >-0.9, which indicates that fines are more mobile than coarse grains because they are sufficiently present on the bed to not be sheltered from flow by coarse grains. (2) Maintain ratios of the median surface grain diameter before spring flow releases (D50,surface) and the average subsurface median grain size (D50,subsurface) both to the median bed load diameter (D50,BL) to respectively produce values of D*surface and D*subsurface that target >1.0. After Paola and Seal (1995), D*surface and D*subsurface ≤1 infer bed load is dominated by surface particles and bar and riffle material and ratios >1 indicate dominance of fine sediment entrained by local scour exposing subsurface sediments and mobilization of fines from channel banks, lee areas, and patches in the channel.

- Localities: Sediment monitoring stations at Lewiston, Limekiln Gulch, and Douglas City.

- Frequency and Timing: In years that sediment mobility is measured, compute b, D*surface, and D*subsurface with bed load and bed material samples and cross section surveys and measured water surface slopes.

- Methodology: For b, see methodology for sediment mobility target. Values of D50,BL are determined by physically sampling bed load at the sediment monitoring stations and dry sieving the material in half-phi size intervals for computing the median bed load diameter by mass for each discharge that samples are taken. Values of D50,surface are measured with a Wolman (1954) sample of 300 grains in the upstream vicinity of the monitoring stations before the spring flow release. Values of D50,subsurface are measured with three or more bulk samples of subsurface sediment in the near upstream vicinity of monitoring stations following criterion in Church et al. (1987) for requisite sample size. The sampling domain for subsurface sediments extends below the depth of the local surface D84.

Channel Migration

- Means Objective: Promote channel migration to increase channel complexity and floodplain development and shift the channel to reset riparian forests by eroding banks and surfaces fossilized by vegetation and course material.

- Fundamental Objective: Increase extent of laterally mobile channel to create new alluvial features, new floodplains, and woody riparian recruitment.

- Targets: “Naturally” construct or further develop meanders with flows and sediment management that have wavelengths, amplitudes, and radius of curvatures predicted with information in the TRRP channel design guide (HVT et al., 2011). Also, laterally shift the Trinity River channel outside meander bends by statistically significant distances every 5 years.

- Localities: Throughout the Trinity River restoration reach.

- Frequency: Every 5 years at any time of year.

Methodology: Utilize geo-rectified aerial photographs to map the channel and determine its change in location since the previous survey. Use continuous lines traced at the wetted summer baseflow to determine vectors of magnitude and direction of change in set increments that are determined through trial and error to best represent the observed shifts in channel position. Estimate error in estimates of shifts in channel position with estimates of horizontal accuracy provided by the contractor for aerial photography. Bin the vectors and their associated error by river miles representing geomorphic provinces in the river and perform t-test to determine whether the observed shifts ±error in channel position are statistically significant. Additionally, use CAD or GIS software to estimate changes in meander properties mentioned and t-tests to determine if the changes are significant or within the range of error. From these analyses, effectiveness meeting targets for channel migration can be evaluated.