Proposed Physical Process Metrics

Active Bar Area

Fundamental Objective: increase active bar coarse sediment storage, increase channel complexity, increase aquatic habitat quantity, quality, and complexity.

Hypothesis for Fundamental Objective: Increasing active bar area is an index for improving a suite of physical and ecological attributes as defined as impaired in the TRFER.

Means Objective: Increase coarse sediment supply and transport rate.

Target – Quantifiable targets will differ by reach and can be developed by evaluating:

- Presence:

  - Are active bars present or not?

  - Should they be present (i.e. do skeletal pre-dam bars exist or is it a transport reach)?

- Magnitude: How does active bar area (per length of channel) compare:

  - between reaches,

  - relative to gravel augmentation locations,

  - relative to channel rehabilitation sites and

  - relative to tributary confluences.

- Trend:

  - Is active bar area increasing?

  - Is active bar area reaching reach-specific targets? Requires a target to be set per Coarse Sediment Storage synthesis report)

Spatial Extent: The 40-mile TRRP reach, divided into geomorphic sub-reaches.

Frequency: Every five years.

Algorithm: The methodology for evaluating the active bar area metric will follow the methods described in “McBain Associates 2015. Trinity River Active Bar Mapping, Lewiston Dam to the North Fork Trinity River Confluence, Summer 2014. Prepared for Hoopa Valley Tribal Fisheries, Hoopa, CA 44 pp.” The field-based mapping methods could potentially be modified to utilize aerial photograph analysis, or a hybrid approach could be developed (office-based air photo mapping followed by field verification). The 2015 report suggests field-based professional judgement is necessary to ensure data accuracy regarding: bar areas covered with overhanging vegetation, grain size criteria, and limiting measurements to bars formed and maintained by post-ROD flows.

Other Items to Consider:

- The merits of the bar area metric are:

  - While some sources of uncertainty must be carefully considered (flow magnitude as “datum” and professional judgement criteria), the method is reasonably repeatable.

  - It requires relatively minimal additional data collection (2 weeks every 5 years).

  - The results are easily interpreted, and the implications are easily understood by a broad audience.

- Active bar area may be an index of coarse sediment storage volume, but more importantly, it provides a stand-alone measure of progress from an impaired state toward a more functional state. It is important to note this is not a direct measure of coarse sediment storage volume, and to acknowledge that local geomorphic/hydraulic changes (incision/aggradation at riffle control) might generate variability. We assume that this variability is small compared to reach totals.

- Active bar area monitoring evaluates a measurable attribute of a scaled-down alluvial river. For example, in response reaches (where bars were historically present) with abundant coarse sediment supply (such as downstream of an augmentation site), bar area might be expected to evolve to a magnitude approximating a maximum post-dam active-bar-area potential. This value might then be used to develop targets for other similar sub-reaches.

- If mapping is conducted during late summer 450 cfs base flow (when tributary accretion is minimal), then the “datum” will be very consistent and will not be a source of year-to-year variability. Flow by reach will be documented per survey.” We have to make the assumption that this will have minimal effect on the results. Perhaps some sort of sensitivity analysis is required – or at least an error estimate from changes in cumulative flow magnitude between survey years.

- Active bars provide fry and juvenile rearing habitat over a range of flows as shown in the TRFEFR (to reverse the “dip in the flow-habitat curve” characteristic of an encroached channel).

- Active bars provide numerous ecological, physical and biological functions aside from providing juvenile salmon habitat (e.g. FYLF breeding habitat).

- Gravel storage can occupy some fish habitat (such as when a holding pool gets filled), but this phenomenon is (1) often short lived (pools tend to re-scour), and/or (2) when scour potential is limited (such as near a dam), a certain amount of coarse sediment may be required to overcome local post-dam deficits and promote ongoing coarse sediment transport. In such cases (as on Clear Creek), the gravel that goes into storage and changes the channel type for short distances, is considered a negligible impact on the overall health and habitat potential of the river.

Bed Mobility and Scour

Fundamental Objective: increase rates of bed mobility and scour in order to increase coarse sediment transport, increase channel complexity, increase aquatic habitat quantity, quality, and complexity.

Hypothesis for Fundamental Objective: Increasing bed mobility and scour is an index for improving a suite of coarse sediment-related physical and ecological attributes as defined as impaired in the TRFER.

Means Objective: Increase coarse sediment (and sub-surface fine sediment) mobility and transport rate.

Target – Quantifiable targets from TRFE Objectives:

- Mobilization of matrix surface particles (D84) on alternate bar surfaces during Normal and wetter water years >6,000 cfs)

- Mobilization of subsurface particles (≥ 1D84 depth) during Wet and Extremely Wet years

- Mobilization of subsurface particles (≥2D84 depth) during Extremely Wet years

Spatial Extent: The 40-mile TRRP reach, divided into geomorphic sub-reaches.

Frequency: All years with Normal or wetter hydrographs.

Algorithm:

Three methods to evaluate bed mobility and scour, along with the pros, cons and relative costs of each are presented below.

- Empirical measurements of bed mobility and scour using tracer rocks, scour cores and scour chains.

  - Pros

    - High degree of certainty from results of direct measurements.

    - Simple methodology, highly repeatable.

  - Cons

    - Spatially limited.

    - Extrapolation to un-measured areas results in higher levels of uncertainty.

  - Relative Cost

    - Medium.

- Predictive statistical model developed from 2009-2014 empirical measurements.

  - Pros

    - Provides estimates of uncertainty

    - Simple, the fieldwork and the model have been completed

    - Facilitates predictions of different peak flow alternatives.

  - Cons

    - The data are dated (2009-2014), may require updating (more fieldwork)

    - Results are limited to areas where monitoring has occurred.

    - Results are sub-divided by reaches and inundation zones but are not spatially explicit beyond reaches and inundation zones.

  - Relative Cost

    - Low (unless mobility and scour data require updating, then Medium)

- Deterministic Model using the SRH2D Hydraulic Model

  - Pros

    - Uses existing data sets

    - Covers a very large area (40 miles), providing systemic predictions of bed mobility and scour.

    - Generates predictions of area mobilized by peak flows, not just along cross sections.

    - Can be overlain with other spatial analyses (e.g. spawning area) to predict ecological impact (e.g. redd scour).

    - Facilitates predictions of different peak flow alternatives.

    - Easy sensitivity analyses of assumptions (critical Shields parameter)

    - Predictions can be analyzed by:

      - Geomorphic reaches (e.g., Lewiston Dam to Rush Creek backwater)

      - Geomorphic feature (e.g., pool tails, point bars, mid-channel bars)

      - Inundation zone (e.g., 450 cfs-2,000 cfs inundation zone on active bars)

      - Mesohabitat boundaries (e.g., pool tails, riffles)

      - Fish habitat suitability boundaries (e.g., Chinook spawning habitat)

      - Riparian vegetation patches (e.g., initiating seedlings along low flow channel margins)

      - Groupings of all the above

  - Cons

    - If physical conditions (grain size, topography) change, the model needs to be updated.

  - Relative Cost

    - Low, unless input data require updating, then High.