YUROK TRIBE SUGGESTIONS FOR PHYSICAL METRICS

(TARGETS AND OBJECTIVES)

Revised December 2021

Scope of Revision

In 2020 the Physical Workgroup solicited proposals for objectives and targets for inclusion in the Program’s monitoring plan. After review of prior objectives, including those put forth in the Integrated Assessment Plan (ESSA 2009), the workgroup settled on a set of objectives to be carried forward to development of targets and inclusion in the Science Plan in the first quarter of 2021. Two metrics proposed by the Yurok Tribe, R* and Aw*, underpinned the objectives intended to increase topographic variability and dynamic inundation with changes in flow.

Both Yurok metrics were subsequently applied in a habitat synthesis report (in review) and an additional report that assesses the performance of the Deep Gulch/Sheridan rehabilitation sites. Application of the metrics revealed the strengths and weaknesses of the metrics as originally described. As discussed at the December 2021 Physical Workgroup meeting, R* was found to be a useful metric in its original form. The original formulation of Aw*, however, is based on somewhat different objective than was appropriate in the context of these reports.

As originally conceived, Aw* would allow comparisons of wetted area as a function of discharge between different reaches of the river and support evaluation of how effective Program actions are at achieving the maximum habitat gains possible in different river reaches or which reaches offer the most potential gains from restoration. Application of the metric, however, showed that the concepts underlying Aw* are also well suited to evaluating temporal changes, or differences between condition (e.g., pre-construction, design, post construction) within reaches, irrespective of what the maximum potential might be. As applied in the habitat synthesis, Aw* was modified to express inundation extent as mean wetted width integrated over discharge (IW), rather than as inundated area integrated over discharge, and attempts to normalize by potential inundation extents were dropped.

The metrics put forth to assess the proposed objectives and targets are robust, highly versatile, and can be applied for many different purposes associated with the Programs management actions. Both have strong relationships to a broad range of channel features that provide habitat for juvenile salmonids and a range of other species. The proposed metrics can also be applied at various spatial scales and can be recalculated from the underlying data if it becomes necessary to modify them in the future. This represents a distinct advantage over many direct measurements that cannot be revised at a later date.

These developments were reported to the Physical Workgroup at its December 2021 meeting, and the workgroup responded by requesting that the previously submitted metrics document be updated to reflect adoption of IW in place of Aw*. That update is provided herein. R* remains the same as previously proposed.

Topographic Variability as Measure by R*

Means Objective: Topographic variability

Hypothesis for Fundamental Objective: Topographic variability is the primary attribute defining spatially complex channel morphology (IAP objective 1).

Target: Increase the value of the metric R*. A target value of R* has not yet been defined, but such a target could be determined by adopting a value representative of reaches that are deemed to be satisfactorily complex. That work has yet to be formally undertaken.

Locations: Throughout the TRRP restoration domain.

Spatial Scale: R* is best applied at the reach scale to river segment when comparing different locations, as small samples of geomorphic units within short reaches can lead to biased results. However, smaller reaches with consistent boundaries can be used when comparing the same reach at different times or under different condition (i.e., design and pre-construction). Analysis should never be used on areas smaller than a complete geomorphic unit (riffle crest to riffle crest) and it is preferred that boundaries coincide with geomorphic controls.

Frequency: Whenever updated bathymetry and topography are available for an area. R* would be computed throughout the project domain approximately every 5 years when full topography is updated, and more often in connection with site-scale survey such as project as-builts or design alternatives.

Reporting: R* will be used to identify both trends and the magnitude of changes toward increased complexity.

Methods: R* is defined in terms of the frequency distribution of flow depths at a reference discharge according to:

R* = [(h75 − h25) + (h90 − h10)] / (h90 + h10)			(1)

where all depth percentiles are based on the reference discharge of 2000 ft3/s. Depths are derived as the difference between the modeled water surface elevation (WSE) at the reference discharge and the terrain models being evaluated. Modeling is performed with SRH-2d. A reference discharge of 2000 ft3/s was selected because that discharge fully inundates the active channel bed. The resulting depths are processed with a Perl script that computes a variety of depth statistics, including the 90th, 75th, 25th, and 10th depth quantiles used in the computation of R*. Depth percentiles are a better choice for assessing topographic relief than ground elevations because the continuous downstream slope of the river means ground elevations would require detrending before use. Depths, on the other hand, are automatically detrended. In addition, the wetted area associated with a reference water surface automatically defines the spatial extent of the “channel” that the metric represents.

Inundation Extent as Measured by Integrated Wetted Width (IW)

Means Objective: Valley/floodplain inundation.

Hypothesis for Fundamental Objective: Wetted area is a key driver of habitat availability at moderate discharges, of fish production, and of riparian health (IAP objectives 2, 3, and 5).

Target: Rehabilitation actions should increase the value of IW. Specific target values of IW cannot be defined because maximum possible values depend on local valley width and similar constraints. Normalization that would support the development of numeric targets are possible, but not complete at this time.

Locations: Throughout the TRRP restoration domain.

Spatial Scale: IW is best applied at the river segment scale when comparing different locations. Ambiguity in the locations of boundaries between reaches become increasingly influential as reach length decreases. However, smaller reaches with consistent boundaries can be used when comparing the same reach at different times or under different condition (i.e., design and pre-construction). Analysis should never be used on areas smaller than a complete geomorphic unit (riffle crest to riffle crest) and it is preferred that boundaries coincide with geomorphic controls.

Frequency: Whenever updated bathymetry and topography area available. IW would be computed throughout the project domain approximately every 5 years when full topography is updated, and perhaps more often in connection with site-scale survey such as project as-builts or design alternatives.

Reporting: IW will be used to assess the success of rehabilitation actions at project sites and temporal trends throughout the Program’s focal area.

Methods: IW is defined in terms of the mean width of inundation integrated over a range of flows, as determined by hydraulic modeling with SRH-2d. Mean wetted width (W) for a given stream reach at a given discharge (Q) is computed as the wetted area at that Q divided by the valley length through the reach. As applied in the habitat synthesis report, wetted width is considered over discharges ranging from 150 to 6000 ft3/s and IW is integrated over discharges ranging from 150 to 2000 ft3/s. As numerically integrated, the general definition of IWR is for a given reach is:

IW= i=1nWi ∆Qi 				(2)

where Wi is the mean wetted width at discharge Qi, DQi is (Qi+1 − Qi-1)/2, Q0 = 0, and n is the number of discrete values of Qi considered. IW can easily be re-computed over alternative discharge ranges as needed to address specific questions. For example, Qn = 3500 ft3/s may be well suited for assessing juvenile rearing habitat availability, whereas Qn = 11000 ft3/s or more might be selected for assessing valley or riparian restoration objectives.