- Riparian Aquifer Storage

- Means Objective: Maintain groundwater storage in riparian aquifers following recharge by high river flows to promote woody riparian recruitment, insect populations and off-channel rearing habitats for salmonids, frogs, and toads.

- Fundamental Objective: Promote biological productivity and woody riparian recruitment on upper bars and floodplains in the Trinity River corridor.

- Targets: Groundwater recession rates should be statistically significantly slower than the local river stage recession. This target could be modified as monitoring data become available.

- Localities: Floodplain aquifers and off-channel depressions (swales, pits, pools, ect.) that are isolated topographically and do not fully drain by surface flow to the Trinity River and are located on floodplains between Old Lewiston Bridge and North Fork Trinity River.

- Frequency and Timing: Every 5 years at any time of year to assess the extent that fine sediment deposition and biomatter accumulation is decreasing hydraulic conductivity in riparian aquifers.

- Methodology: Establish a local benchmark and install rebar in monitored depressions and the low flow channel of the Trinity River. Survey the tops of rebar and piezometers installed in the aquifer relative to the local benchmark. Closely monitor water levels in the depressions, aquifer, and river channel during spring flow releases using measurements of water surface levels taken down from the rebar tops or piezometer tops or with pressure transducers to determine river discharges and rates of inundation and recession for the depressions and aquifer by surface and subsurface flows. These data will enable determination of off-channel rearing habitat relationships to discharge in depression features and lag times for aquifer drainage as a function of discharge, location on the floodplain, and river mile. The lag times can be related to aquifer properties, including porosity and hydraulic conductivity for assessing substrate conditions for plants and water storage in floodplain areas.