Meeting Summary

FLOW WORKGROUP

Monday March 2, 2026 9:00 – 12:00

Google Meet/Willow Creek screw trap office (YT)

## Purpose

- Decide which hydrographs to forward to IDT/TMC

## Action Items Derived During the Meeting

- Patrick will follow-up with modelers for one-pagers on their methods

- Patrick will follow-up with Smokey for the bedload modeling

- Patrick will write the IDT memo

- Eric will work out the subdaily (diurnal) flows

## Meeting Summary

### Flow Workgroup Meeting Summary

March 2, 2026

Attendees: Patrick Flynn (coordinator) Simone Groves (HVTF), Chris Laskodi (YTF), Derek Rupert (USBR), Jacqueline Bridegum (USFWS), James Lee (USBR), Eric Peterson (USBR), Seth Naman (NMFS), Shane Quinn (YTF), Steve Gough (USFWS), Justin Alvarez (HVT), John Bair (ARS-HVT), Don Ashton (ARS-HVT), Ken Lindke (CDFW), Seth Naman (NMFS), Justin LaNier (DWR)

## 1. Hydrologic Conditions

- Review of March B120 and CNRFC forecasts indicates lower-end normal water year conditions.

- Snowpack is low; concern raised about whether models fully capture amplified snowmelt under warm spring conditions.

- Reclamation expects Trinity Lake to fill, with ~½ normal diversion this summer.

- A 6,000+ cfs synchronization flow occurred in December.

## 2. Hydrograph Modeling Results

### Fish (S3 Model)

- Nearly identical results across hydrographs (A vs. B) and dry vs. normal years.

- Increasing consumption rates slightly increased fish output but did not differentiate scenarios.

- Temperature likely limiting differentiation (RBM10 inputs unchanged).

### Temperature Modeling

- Very few exceedances across alternatives.

- Minimal differences between scenarios; slight 7DADA variation in June pulse under A, not meaningful.

### Riparian Vegetation (TARGETS)

- Hydrograph A produced more seedlings than B.

- No encroachment risk, even under Dry-Dry scenario, due to December 6000+ cfs flow.

- Black cottonwood modeled per synthesis report:

  - Wide dispersal window

  - Typically above 2000 cfs elevation

  - Fast-growing, high habitat value

- “Bathtub ring” (450–2000 cfs capillary fringe) discussed.

- Receding limb shape and site construction (low surfaces) are key controllable factors.

- Early snowmelt recession may favor red willow and arroyo willow establishment.

### Frogs (FYFAM Model)

- Hydrograph A performed better overall.

  - Often double (or more) survival to froglet stage compared to B.

  - Dry-Dry A scenario performed particularly well.

- Hydrograph B had slightly earlier metamorphosis (7–10 days earlier), but difference not biologically meaningful.

- Key drivers: desiccation timing and egg deposition timing.

- Roughness before peak discourages premature breeding.

- Dry years benefit mainstem frogs; tributaries may act as sinks.

### Inundation Model

- Results largely similar.

- Slight (<10%) advantage to Hydrograph B, not materially different.

### Sediment Transport

- Current year results pending.

- Expectation that A would move more sediment.

- Sediment objectives largely met with December sync flow.

- Sediment not a primary objective this year.

## 3. Broader Discussion Points

- Limited flexibility for fish objectives in a dry-ish year.

- Greater opportunity to target riparian and frog objectives.

- Recognition that newer Objectives and Targets should guide decisions (though emphasis similar to Flow Study objectives).

- Desire to better understand mechanistic drivers behind model results.

- Consideration of shifting peak of A earlier and lowering magnitude, but constrained by April B120 publication and process requirements.

## 4. Fall Pulses & Hydrograph Shape

- Lack of fall pulses noted.

- Historic hydrographs show late-summer/early-fall pulses are rare.

- Fall pulses would likely require separate NEPA analysis.

- Discussion of snowmelt patterns, geomorphic context, and recession rates.

## 5. Sub-Daily & Diurnal Flows

- Recommendation to hold peak flows at average levels (e.g., ~8000 cfs), not exceed.

- Diurnal snowmelt pattern discussed:

  - Infrastructure retrofitted to allow implementation.

  - Considered ecologically important to mimic natural signals.

- Action item: Eric to check with CVO on diurnal fluctuation options.

- Reference made to Todd Buxton whitepaper on diel fluctuations.

- Simone mentioned Todd Buxton’s whitepaper on diel fluctuations: https://www.trrp.net/library/document/?id=2648

## 6. Decision

✅ Consensus reached to move forward with Hydrograph A to IDT/TMC.

Rationale:

- Stronger frog survival outcomes.

- Better riparian recruitment.

- Comparable fish and temperature outcomes.

- Sediment objectives already largely met.

- Differences in inundation and metamorphosis timing not substantial enough to outweigh A’s benefits.

### Key Takeaway

Given dry-leaning conditions and sediment objectives already achieved, the group prioritized riparian and frog benefits, resulting in selection of Hydrograph A for advancement.

Next meeting: TBD.

Adjourned 12:00.