# Meeting Summary

# Physical Workgroup 3-20-13 Call

# Agenda

9:00	hatchery gravel goals

9:30	2013 flow schedule

9:45 	FY14 workplan prioritization

10:30 	adjourn

# Workgroup Recommendations:

Hatchery Coarse Sediment Augmentation Goals:

Over the long term, add coarse sediment at a rate equal to transport.

2013 Flow Schedule

Update the existing write-up for proposed WY13 geomorphic peak release alternatives to explicitly mention:

- The flow recommendations for higher magnitude, shorter duration peak release in WY13 is only for this water year and does not apply to out years.

- Sediment monitoring since 2004 has shown that transport rates are nearly an order of magnitude less than predicted by the 1999 Trinity River Flow Evaluation Report.

- Specific objectives for the higher magnitude flow releases include:

  - Mobilize Rush Creek Delta sediments.

    - The large storms in December 2012 aggraded Rush Creek delta by ¾ foot (as measured by the water surface elevation at the crest).

  - Maintain the Upper Junction City site.

    - The site was just built last summer and could use higher flows to:

      - Flush out the remnants of the native fine sediment in the constructed side channel on river left with the infiltration gallery at the head.

      - Flush out coarse sediment in the upper split flow channel that deposited during a brief winter storm that caused some bank adjustment but was too short in duration for the eroded material to clear out of the split flow channel.

FY14 Workplan Prioritization for the Physical Workgroup Proposals

The workgroup recommends the FY13 workplan and priorities for physical workgroup proposals carry over to FY14. No new items are proposed. The workgroup requests the opportunity to review and revise this recommendation once a complete list of the FY14 workplan for the whole program is available.