FY 2012 Riparian Monitoring should evaluate:

1.	How ROD releases and channel rehabilitation site designs lead to successful germination, initiation, and establishment of riparian vegetation on various alluvial and depositional features, including modes of mortality.

2.	How riparian plant habitat evolves and is used by riverine and riparian associate terrestrial species (e.g., birds, Foothill Yellow-legged Frogs, Western Pond Turtles, and).

3.	Whether mature plants are contributing to the large wood supply.

Proposed performance measures

Program success is very dependent on preventing large-scale detrimental riparian encroachment (Objective 5.2) while encouraging riparian establishment on a wide variety of geomorphic surfaces for fish and wildlife habitat (Objective 5.1). Therefore, performance measures must track the success of managed streamflows, bank rehabilitation, and sediment augmentation at preventing detrimental riparian encroachment in the 450-2,000 cfs inundation zone; they should also provide timely information to the adaptive management process about whether riparian vegetation is approaching a threshold beyond the control of ROD management actions, thereby initiating an irreversible decline in habitat quality, quantity, and availability.

Key system-scale performance measures should include:

•          patch type frequency, abundance, and spatial and size distribution within a consistently defined riparian corridor; and

•          unvegetated cover type frequency, abundance, distribution, and area within a consistent definition of riparian corridor.

The key site-scale performance measures should include:

•          patch type frequency, abundance, and spatial and size distribution at GRTS selected sites within a consistently defined riparian corridor;

•          area, frequency, and abundance of colonizing and established riparian patches between 450 cfs and 2,000 cfs;

•          plants species frequency, abundance, and distribution at bank rehabilitation sites within inundation zones;

•          age class distribution;

•          region of river bank above summer baseflows where woody riparian plant species are initiating and establishing; and

•          zones and abundance of frequently sampled riparian plant species.