TRINITY RIVER RESTORATION PROGRAM CHANNEL REHABILITATION MONITORING PROPOSAL SUMMARY

Monitoring Proposal Title:

Identifying factors that optimize riparian revegetation success in meeting compensation and habitat replacement goals in the shortest amount of time.

Proposal Submitted by:

Hoopa Valley Tribe Design Team

Restoration Goal: Briefly state goal of Channel Rehabilitation Project element(s) that is(are) to be monitored

The revegetation goal is to add channel roughness, increase plant species richness, and to compensate for construction related aquatic and wildlife habitat impacts at channel rehabilitation sites, all in support of river health – a TRRP Fundamental Objective.

Monitoring Question: Explicitly state monitoring objective into testable question or hypothesis

Hypothesis 1: Planting survival is more dependent on cultural methods than hydrologic and physical characteristics within a site. Hypothesis 2: Survival is dependent on the type and size of material planted and the season during which material is planted. Hypothesis 3: Planted species richness and aquatic and wildlife habitat characteristics are greater at 5 years in higher cost plantings, which accomplish compensation goals faster (1:1 replacement). Hypothesis 4: Vertical and areal growth rate is dependent on irrigation frequency and quantity, planting maintenance, and planting protection.

Monitoring Design Plan: Describe or summarize proposed monitoring plan in one page or less. Identify the channel rehabilitation project properties that are to be measured and proposed methods. Identify how monitoring addresses Monitoring Question. Attach additional information to the proposal summary if desired.

Overview: The project will compile existing riparian planting survival and growth data into one database to conduct subsequent analyses (e.g., evaluating factors affecting planting survival). Spatial analysis will rely on existing topographic and hydrologic data. Preliminary analysis will focus on identifying which factors most affect plant survival and growth (e.g., species, material type, material size, time of year planted, irrigation, mulch, plant protection, age, design element, height above river, and distance to channel margin). The study will also identify the types of design elements most closely associated with riparian planting success. These design elements could be applied in future designs. Project priorities and proposed methods: - Site identification criteria (n=7): Seven sites with as-built, survival, and growth data collected through 2014 will be assessed, specifically: Sawmill (2009), Reading Creek (2010), Trinity House Gulch (2010), Lowden Meadows (2010), Wheel Gulch (2011), Upper Junction City (2012), and Lower Steiner Flat (2012). Additional data collection is not required at these sites to complete this analysis. - Evaluate revegetation components at each site, including: - Number of species planted vs. surviving, - Percentage of individual species planted vs. surviving, - Species survival across all sites and individual sites , - Material type survival across all sites and individual sites by species, and - Vertical and areal growth as a function of irrigation, mulch, maintenance, protection, age, (Plant care will be care will be quantified on a scale of 0 through 5, with 0 = plant and pray and 5 = weeding, mulch, plant protection, lots of irrigation.) - Analysis will evaluate: - Which revegetation species are most likely to survive? - Whether planting location or design element influences planting survival. Location or design element may be used as proxy for soil and water availability. - Whether increased revegetation cost leads to better survival and growth at 1, 3, and 5 years. - Which combination of species, material types, and maintenance/irrigation intensities will best meet our construction impact compensation requirements of 1:1 replacement and no net wildlife habitat loss? - Study conclusion will recommend refinements to artificial planting methods to improve effectiveness, including considerations for overall cost and maintenance post-planting.

Monitoring Locations and Number of Sites: Identify Projects and locations within Projects to be sampled

See site identification information above. In total, 7 sites will be sampled.

Monitoring Frequency: Identify when sampling will be performed, frequency (e.g., one-time, annually for five years)

One time initially to develop study results and recommendations. We recommend conducting a repeat investigation three to five years later as needed.