TRINITY RIVER RESTORATION PROGRAM CHANNEL REHABILITATION MONITORING PROPOSAL SUMMARY

Monitoring Proposal Title:

Assessment of side-channel design parameters and their influence on resulting performance.

Proposal Submitted by:

Hoopa Valley Tribe and Yurok Tribe design teams

Restoration Goal: Briefly state goal of Channel Rehabilitation Project element(s) that is(are) to be monitored

The goal of constructed side-channels is to create large scale and immediate improvements in aquatic habitat quantity and quality for fry and juvenile salmonid rearing during all baseflows. Longevity of these features, and their ability to assist in maintaining long-term river health (a TRRP Fundamental Objective) is at question. As observed since the mid-1990s, many low flow side-channel entrances are filling with sediment, while still functioning at higher flows. The performance and cost-effectiveness of low flow side channels could be improved (longevity) by revising design guidelines in accordance with findings of this effort.

Monitoring Question: Explicitly state monitoring objective into testable question or hypothesis

Hypothesis 1: The underlying channel type of the Trinity River under the ROD flow and sediment regime does not support long-term multiple channel morphology (CDG 2011, Shea 2013). However, the likelihood of longer-term persistence of multiple channels can be increased based on using side channel entrance design criteria summarized in the CDG, and to focus on those reaches that may be more prone to self-maintenance (http://odp.trrp.net/Data/Documents/Details.aspx?document=1906 ). Hypothesis 2: Constructed side-channels with entrance conditions designed following Channel Design Guide (CDG) criteria have greater longevity compared to side-channels constructed without applying these criteria. Hypothesis 3: Side-channels designed where the entrances (upper 1/3 of side-channel) can transport coarse sediment and located on the downstream outside portion of meander bend will be more likely for self-maintenance over the long term. Hypothesis 3: Side-channels designed with large wood structures at the entrances will be more likely for self-maintenance over the long term.

Monitoring Design Plan: Describe or summarize proposed monitoring plan in one page or less. Identify the channel rehabilitation project properties that are to be measured and proposed methods. Identify how monitoring addresses Monitoring Question. Attach additional information to the proposal summary if desired.

Overview: This project will test the influence of side channel design parameters (CDG 2011) for side channels constructed as part of Phase 1 implementation. The assessment will evaluate CDG criteria by identifying a subset of constructed side channels and evaluating their performance since construction. We will use existing topographic information (with supplementation if needed) to evaluate w/respect to CDG criteria, then relate to side channel entrance evolution (i.e. self-maintenance). As data allow, the physical parameters for side channels that have been blocked with sediment, as well as those with greater longevity, will be described. Design criteria in the CDG can be updated accordingly. Proposed methods: - Identification of side channels with at least one high flow > 8,500 cfs (n=10-20) - Gathering of topographic information and SRH 2-D hydraulic model output at each site - Documentation and evaluation of physical variables including side channel length, slope, wood volume, streamflow, and entrance condition - as appropriate. - Analysis and reporting: - Does the site support a multi-channel morphology based on the analysis recommended in the CDG (2011) and Shea (2013)? - Are side channels that satisfy CDG and multi-channel morphology criteria persisting or failing (entrance remaining open)? - What physical parameters (length, slope and large wood dimensions) most contribute to self-sustaining side channel entrances? Results may be applied to future site designs and will also enable the Design Work Group to update CDG guidelines as appropriate.

Monitoring Locations and Number of Sites: Identify Projects and locations within Projects to be sampled In consultation with the Design Team, 10-20 side channels will be selected to represent a diversity of side-channels that have maintained themselves as well as those that have filled.

Monitoring Frequency: Identify when sampling will be performed, frequency (e.g., one-time, annually for five years)

Sampling will performed once, during FY2016. If CDG is updated based on results of evaluation, it is recommended that newly implements sites (using new guidelines) be evaluated within three to five years to assess performance improvements and adaptive management purposes.