TRINITY RIVER RESTORATION PROGRAM CHANNEL REHABILITATION MONITORING PROPOSAL SUMMARY

Monitoring Proposal Title:

Assessment of forcing feature performance on active bar formation

Proposal Submitted by:

Hoopa Valley Tribe Design Team

Restoration Goal: Briefly state goal of Channel Rehabilitation Project element(s) that is(are) to be monitored

Adding coarse sediment and conducting channel rehabilitation under the ROD is intended to increase and maintain coarse sediment storage in the reaches downstream of Lewiston, as well as increase fluvial processes that create and maintain high quality aquatic habitat while supporting river health (a TRRP Fundamental Objective). The effectiveness of these actions can be increased by integrating with “forcing features.” Forcing features are physical elements added to a channel to facilitate a physical response, and have been incorporated into Trinity River designs for a variety of purposes, including maintaining side-channel or split channel entrances, encouraging channel migration, and encouraging formation of active bars (e.g., point bars, medial bar, or lee deposit). “Key piece” forcing features (large wood or large boulders) are often used, but to date the effectiveness of these features has not been assessed.

Monitoring Question: Explicitly state monitoring objective into testable question or hypothesis

Hypothesis 1: The number of large wood pieces and/or front cross section area is directly related to gravel bar size. Hypothesis 2: The spatial arrangement of large wood pieces and other roughness elements within the planform is related to gravel bar formation and size. Hypothesis 3: Channel setting (e.g., slope, grain size, physical dimensions, channel expansions and constrictions, and radius of curvature) can influence performance of forcing features. The Channel Design Guide (2011) and Active Bar Mapping project (2015) have provided initial assessments of these relationships, but additional evaluation of the hypotheses is needed to better inform future designs.

Monitoring Design Plan: Describe or summarize proposed monitoring plan in one page or less. Identify the channel rehabilitation project properties that are to be measured and proposed methods. Identify how monitoring addresses Monitoring Question. Attach additional information to the proposal summary if desired.

Overview: We will evaluate channel response to a range of forcing features (constructed large wood and boulder structures) relative to stated design objectives. Evaluations will include 1) examining the number of large wood pieces relative to gravel bar size (a continuation of preliminary analyses by the Active Bar Mapping project in 2015), and 2) identifying slope, approach, grain size, channel dimensions, and curvature associated with selected forcing features. Project properties and proposed methods: - Evaluations at each site: - Number of large wood pieces - Dimensions of large wood/boulder structure, including front cross sectional area - Dimensions and grain size of gravel bar upon which forcing feature is placed (if applicable) - Channel slope - Current state of channel expansion/constriction within proximity of forcing feature - Physical response to the forcing feature, including bar area, channel migration, local bathymetric changes compared to the as-built condition. - Data collection and data sources: Large wood data available from annual surveys by USFWS, HVT, and the Active Bar Mapping project. Structure dimensions measured in the field and/or available from TRRP as-built surveys and associated documentation. Channel slope data are available from existing DTMs. Any additional data needs would likely be minor, developed as needed. Results will be applicable to future designs (construction sites, gravel augmentation and large wood augmentation0 and will also enable the Design Work Group to update CDG guidelines as appropriate.

Monitoring Locations and Number of Sites: Identify Projects and locations within Projects to be sampled

We will, in coordination with the Design Team, select a sample representing the range of forcing features for evaluation. At least 10 replicates of each type would be desired (large wood structure, boulder structure, channel expansion).

Monitoring Frequency: Identify when sampling will be performed, frequency (e.g., one-time, annually for five years)

Analysis will performed once, in 2016. Certain forcing features may have few replicates or a too-short response time since construction, so there could be a recommendation for revaluation again within three to five years for adaptive management purposes.